# DHM ↔ SAPPHIRE Flow — data-format confirmation checklist

To: DHM data team   From: hydrosolutions (SAPPHIRE Flow)   Date: 2026-07-13

Thank you for the sample files (daily flow, staff-gauge readings, rating table) and the QC document. To build the automatic link that reads DHM data into the SAPPHIRE forecasting system, we need to confirm a few practical details about how the real data will reach us and exactly what each field means.

How to use this list: please fill in the "Your answer" column. Short answers are perfect — a "Yes/No", a number, or one line is enough. "Not sure — will check with responsible staff" is a completely fine answer; just mark it so we know to follow up. Items marked ⭐ are the ones we need first in order to start.

This list covers two things: (a) the format of the data you will send us, and (b) the station metadata / GIS we need from you to set each station up (§7).

## Our current understanding — please confirm or correct

We think the setup is as follows. If any line is wrong, please correct it — that alone answers many of our questions.

- Operational (real-time) data reaches us via an API from your database.

- The API gives us water level (gauge height, in metres); we convert level → discharge ourselves using the rating table.

- We produce forecasts only for automatic (telemetric) stations, which send water level roughly every 10–15 minutes.

- Historical data (daily discharge, past manual staff-gauge readings, and rating tables) reaches us as files.

- Rating tables are uploaded periodically; each has a validity period (From/To date), and we apply the table valid for each reading's date.

Is the above correct?   Yes / No — corrections: ____________________

### The samples, as we received them (for reference)

Daily flow (historical) — DFL_…txt

Daily Flow of Station: A in m3/s
 2011
01/Jan/2011,365
02/Jan/2011,364

Staff-gauge readings (historical manual level) — GHT_…txt

Staff Gauge Readings of Station: A in m
 2011
01/Jan/2011,08:00,2.86
01/Jan/2011,12:00,2.80
01/Jan/2011,16:00,2.80

Rating table — RT_…txt

Station No: A
Data Type: Rating Table
Left Column= Stage in m
Right Column= Discharge im m3/s
Rating Type No =  8
From Date = 21-Jul-2011
To Date = 15-Jul-2012
From Stage = 1.0
1,59.6
1.1,70.2

Question: Can we assume that we receive this exact format of the data in .txt files with these headers?

Your answer: __________________

## 1. Station types & scope

Why we ask: this decides which stations we forecast, which data channel each one uses, and how it reaches our software.

| # | What we need to know | Example of the answer we're hoping for | Your answer |
| --- | --- | --- | --- |
| 1.1 ⭐ | Confirm we produce forecasts only for automatic (telemetric) stations, not manual staff-gauge stations. | Yes / No (if no: ____) |  |
| 1.2 ⭐ | Automatic stations send water level every how many minutes (10? 15? varies by station?), and does the API serve exactly this? | e.g. "10 min for all" / "10–15, varies" |  |
| 1.3 ⭐ | Are manual (staff-gauge) stations also available operationally via the API, or do they only ever come as historical files? If we ever needed manual data operationally, how would it reach our software? | e.g. "manual = files only, not on API" |  |
| 1.4 | For the stations in scope, which are automatic vs manual? (a list, or a pointer to where we can see it) | e.g. "Stns 439, 447 automatic; 601 manual" |  |

## 2. The operational API (automatic stations)

Why we ask: this is our main live input; we need the access details, exact meaning of each value, and time reference.

| # | What we need to know | Example answer | Your answer |
| --- | --- | --- | --- |
| 2.1 ⭐ | Confirm the API returns water level in metres (not discharge). | Yes / No |  |
| 2.2 ⭐ | API access + a dummy example. The web address (URL), how we authenticate (key / token / login), and how we request a station + time range (or "latest"). We understand real access can't be granted yet — even a sample API command for a dummy / test station (no real access code needed) plus an example response would let us start building. A link to API docs is ideal. | e.g. GET https://…/level?station=DUMMY&from=…&to=… + a sample response |  |
| 2.3 ⭐ | Can the API be queried for past data (a historical date range), or only the latest reading(s)? (This decides whether we can backfill history via the API or need files — see 3.1.) | e.g. "any date range" / "latest 24 h only" |  |
| 2.4 | Response format: JSON / CSV / plain text? Does it include any header/metadata (station id, unit, timezone)? | e.g. "JSON: [{time, value}]" |  |
| 2.5 ⭐ | Time zone of the API timestamps. | Nepal local time (UTC+5:45) / UTC / other |  |
| 2.6 ⭐ | Datum: is the level measured from the staff-gauge zero at the station, or metres above mean sea level (amsl)? Does it differ by station? (please list amsl stations) | e.g. "gauge-zero for all" / "Stn 601 = amsl" |  |
| 2.7 | Freshness: how soon after a measurement is it retrievable, and how often should we poll? | e.g. "available ~2 min later; poll every 10 min" |  |
| 2.8 | Missing / removed data: see §5 — please confirm flagged values are simply absent from the API response. | (answer in 5.1) |  |

## 3. Historical files

Why we ask: we need history to train the models, at the same resolution we will forecast.

| # | What we need to know | Example answer | Your answer |
| --- | --- | --- | --- |
| 3.1 ⭐ | For training an automatic station we need its historical high-frequency (10–15 min) water level. Can we obtain this by querying past data from the API (2.3), or is it available only as files? If files, in what format and resolution (10-min like the API, or the 3×/day manual GHT layout)? | e.g. "API serves history from 2019" / "files, 10-min" |  |
| 3.2 ⭐ | Daily flow file (DFL): unit always m³/s? Values integer or decimal? How is a missing day shown (blank / -999 / row absent)? | e.g. "m³/s, integer, row absent" |  |
| 3.3 | How is daily flow defined? Daily mean / instantaneous / max? From mean stage → rating or mean of instantaneous Q? Day boundary in NPT? Is it provisional or final, and what tolerance should we expect vs our own level→Q conversion? | e.g. "daily-mean stage → rating, final, ±2%" |  |
| 3.4 ⭐ | Delivery — our proposal: we set up a drop location on our server (e.g. an SFTP folder or shared drive) where you deposit new files (daily flow, rating tables) as they become available — we assume roughly once a year. Does that work, what cadence, and how will we know a file is complete (a done-marker / checksum) and the rule when a file replaces an earlier one? | e.g. "annual; a .done marker; new file replaces old" |  |
| 3.5 | File grammar & naming: filename convention (do prefixes DFL/GHT/RT identify type?), one file per year or multi-year, text encoding, and any missing-value marker inside a file. | e.g. "<PREFIX>_<code>_<year>.txt, one year, UTF-8, -999" |  |
| 3.6 ⭐ | Coverage manifest: for each station in scope, the earliest & latest data, the cadence, and any known gaps — so we know what history we actually have to train on. | e.g. "Stn 439: 2015–now, 10-min, gap 2018-06" |  |

## 4. Rating tables (level → discharge conversion)

Why we ask: we convert every live level reading to discharge using these, so we need the exact table to use, how updates arrive, and the conversion rules.

| # | What we need to know | Example answer | Your answer |
| --- | --- | --- | --- |
| 4.1 ⭐ | The currently-active rating table we should use for live conversion — what format is it, how is it delivered (a file like the RT sample?), and does its To Date stay open / in the future until a newer table replaces it? | e.g. "same RT file format; latest has To Date far in the future" |  |
| 4.2 ⭐ | New tables are uploaded periodically — how do we receive them (same folder/format), how do we recognise a new/updated one, and do we correctly select by From Date/To Date? | e.g. "new file in same folder; pick by date window" |  |
| 4.3 ⭐ | Do validity periods ever overlap or leave gaps? If two tables overlap, which wins (highest Rating Type No? most recent upload?)? If there's a gap, what should we do? | e.g. "no overlaps/gaps by design" |  |
| 4.4 ⭐ | Out-of-range levels — below From Stage (1.0 m → 59.6 m³/s) or above the top row (14 m): what should we do? In particular, what discharge applies for a level below 1.0 m? | e.g. "clamp to ends" / "below 1.0 m ≈ 0 flow" / "flag" |  |
| 4.5 | Interpolation between the 0.1 m steps — is straight-line (linear) acceptable, or is there an exact equation we should use? | e.g. "linear is fine" / "Q = c(h−a)^b, params: ___" |  |
| 4.6 | Corrections beyond a simple shift: are there ever temporary shifts, segmented / compound rating curves, or an underlying equation (e.g. Q = c(h−a)^b) behind the table — and how/when are these applied to the level? | e.g. "occasional temporary shift, provided per period" |  |
| 4.7 | Confirm rating units: stage in m, discharge in m³/s. | Yes / No |  |
| 4.8 ⭐ | Versioning: is Rating Type No a stable, unique per-station version? Can a table be revised in place (same number, changed values), backdated, or reissued for a period already covered? (We need this to detect and re-apply updates correctly.) | e.g. "unique & immutable; new number for any change" |  |
| 4.9 ⭐ | Boundary timing: is the rating To Date inclusive? When a rating changes on a date, does the new table take effect at the start of that day (NPT)? (Decides which table converts a reading right at the boundary.) | e.g. "To Date inclusive; new table from 00:00 NPT" |  |

## 5. Quality control (QC) & corrections

Why we ask: the QC document says flagged data is not sent, which changes how we read missing values; and corrections may arrive later.

| # | What we need to know | Example answer | Your answer |
| --- | --- | --- | --- |
| 5.1 ⭐ | Confirm flagged data is not transferred via the API — so a flagged value simply arrives as a gap, with no marker (we can't tell it from a sensor outage). | Yes / No |  |
| 5.2 | Do the automatic-station levels get QC-checked (Threshold / Step) in DMS? Are thresholds defined per station, per variable (one rule for all water-level stations), or per station and variable? Can you share the actual threshold and step limits for water level at our stations? | e.g. "per variable; WL 0–15 m; step ±2 per 10 min" |  |
| 5.3 ⭐ | How do we learn about later corrections and deletions? If a value is later accepted/modified/deleted in DMS, does the change show up via the API, and how do we detect it — a "modified-since" query, a per-value last-updated / version, a deletion signal (or does the value just vanish)? And how far back should we re-check for changes? | e.g. "modified-since endpoint; deletes leave a tombstone; re-check 30 days" |  |
| 5.4 | Which QC flag names exist? (the dialog showed "Erroneous") | e.g. "Erroneous, Suspect, Manual" |  |

## 6. Station identifiers & other parameters

| # | What we need to know | Example answer | Your answer |
| --- | --- | --- | --- |
| 6.1 ⭐ | We understand the real station IDs may not be assigned/selected yet — can you confirm their format (likely numeric codes?), and that the IDs, names, and any official WIGOS ids will be provided once the stations are chosen? | e.g. "numeric codes, provided at station selection" |  |
| 6.2 | Besides water level / discharge, will DHM also provide precipitation/rainfall (mentioned in the QC document) or other parameters? For which stations, at what frequency, via API or file? | e.g. "rainfall at Stn 439, 10-min, API" |  |
| 6.3 | Are all dates Gregorian (as in the samples), or is Bikram Sambat used anywhere in the data we'd receive? | e.g. "all Gregorian" |  |
| 6.4 | If convenient, one station list/spreadsheet covering: id ↔ API code, automatic/manual, coordinates, datum, river/basin, status, measured parameters, and any upstream flow regulation (dams/diversions) that affects the gauge. | e.g. "one CSV with these columns" |  |

## 7. Station metadata & GIS we need from you (for station setup)

Why we ask: to set up each forecast station we need its location, its vertical reference (datum), and its catchment. Where something isn't available, we can derive or defer it — we just need to know which case applies.

| # | What we need you to provide (per station) | Example / expected form | Your answer |
| --- | --- | --- | --- |
| 7.1 ⭐ | Gauge coordinates — latitude & longitude (WGS84 decimal degrees). Please flag any station whose coordinates are unknown or may be incorrect — we will investigate those separately and treat them as lower priority, so they don't hold up the rest. | e.g. "27.8768, 87.1520" ; "Stn 601 — coordinates uncertain, flag" |  |
| 7.2 ⭐ | Datum (gauge-zero elevation) in metres above mean sea level (amsl) — the elevation of the gauge's zero point. Is this available as station metadata? We need it to relate gauge-height readings to amsl (and for any station that reports level in amsl — see 2.5). | e.g. "yes, per-station amsl datum available" / "not available" |  |
| 7.3 | Catchment / basin outline (drainage-area polygon) for each gauge — available as shapefile / GeoJSON, in which coordinate system (CRS/EPSG), keyed by the station code? If not available, we will derive it from a public DEM — just confirm none exists so we plan for that. | e.g. "shapefiles, EPSG:4326, keyed by code" / "none — derive" |  |
| 7.4 | Catchment area (km²), if known — helps us validate any outline we derive. | e.g. "≈ 3,700 km²" |  |
| 7.5 | Any other station metadata you hold: station elevation, river name, sensor type (radar level sensor / staff gauge), operational status (active/inactive). | e.g. "radar sensor, active, Sapt Koshi" |  |
| 7.6 ⭐ | Station lifecycle history: for each station, any relocations, gauge-zero (datum) changes, sensor replacements, or station-ID reuse — and whether any of these reset the rating curve. (A datum change or relocation silently invalidates older level→discharge, so we must know the dates.) | e.g. "Stn 439 datum revised 2016-05; rating reset then" |  |

## 8. For your API & database team (more technical)

Why we ask: you mentioned there is no API documentation yet, so we need your developers to spell out how the API behaves — this is what lets us build a reliable, correct connection. The contact person can pass this whole section to the API / database team.

| # | What we need to know | Example answer | Your answer |
| --- | --- | --- | --- |
| 8.1 ⭐ | API contract (since no docs exist, please spell it out): endpoint(s), query parameters, the response schema, whether results are paged (and how), the maximum time window per request, result ordering, and how duplicate timestamps are handled. | e.g. "1 endpoint; JSON; paged 1000/req; max 31 days; sorted ascending" |  |
| 8.2 | Errors & limits: HTTP status/error codes, any rate limits, request timeout, and retry guidance; can we request several stations in one call? | e.g. "429 above 60/min; retry with backoff; one station per call" |  |
| 8.3 ⭐ | Time precision: exact timestamp format, the UTC offset (NPT = +05:45), seconds precision, and the definition of a local day (needed for daily aggregation). | e.g. "YYYY-MM-DD HH:MM:SS+05:45, minute precision" |  |
| 8.4 | Bootstrap & handoff: how far back the first API backfill can go, the date from which we use API vs files, how a partial current day is served, and which source wins if a timestamp appears in both a file and the API. | e.g. "API from 2019; API wins on overlap" |  |
| 8.5 | Environments & auth operations: is there a test/sandbox environment; IP allow-listing; TLS/cert requirements; token expiry & rotation (who owns it); any planned maintenance windows. | e.g. "sandbox available; IP allowlist; 90-day token" |  |
| 8.6 | Edge-case examples (very helpful): could you share small sample files / API responses showing the tricky cases — a missing value, a duplicate, a revised and a deleted value, an out-of-range level, and a replaced / overlapping rating table? | e.g. "attached: 6 small examples" |  |

Thank you! Even partial answers help us move forward. Please send back this table with the "Your answer" column filled in, and flag anything that needs a senior colleague so we can arrange a short call if useful. Sections 1–7 can be answered by one person; section 8 is for your API/database team.